Rbd 4/6/26

Step by Step instructions on verifying reads and writes to dual port memory with XSCT debugger

- After installing tools on linux and building the with the xsa in vitis we are ready to proceed with

Debugging the dual port memory

- Open up a terminal

- $: source /tools/Xilinx/Vitis/2022.2/settings64.sh

- hw_server

- Open up another terminal in linux

- At bash $: export TERM = xterm

- $: source /tools/Xilinx/Vitis/2022.2/settings64.sh

- $: xsct

- connect

- On terminal 1

- $: jtag targets ( Find the one that references PL)

- Pick PL target

- $: fpga -file /home/path/to_your/bitfile.bit

- Make the PMU MicroBlaze visible

- $: targets -set -filter {name =~ "PSU"}

- $: mwr 0xffca0038 0x1ff

- $: after 500

- Confirm MicroBlaze PMU now appears

- $: targets

- Load and run PMU firmware

- $: targets -set -filter {name =~ "MicroBlaze PMU"}

- $: dow /home/ray-duran/workspace/zcu104_hw_dpmem_platform/export/zcu104_hw_dpmem_platform/sw/zcu104_hw_dpmem_platform/boot/pmufw.elf

- $: con

- $: after 500

- Reset, load, and run FSBL on A53 #0

- $: targets -set -filter {name =~ "Cortex-A53 #0"}

- $: rst -processor

- $: dow /home/ray-duran/Desktop/zcu104_hw_dpmem_platform/export/zcu104_hw_dpmem_platform/sw/zcu104_hw_dpmem_platform/boot/fsbl.elf

- $: con

- Give FSBL time to initialize the PS/DDR/interconnect

- $: after 5000

- Halt the A53 so XSCT is back under debugger control

- $: stop

- Retarget the A53( should already be there, this is just being cautious)

- $: targets ( Fond one that targets A53 #0)

- $: targets 10 ( As an example)

- $: memmap -addr 0xA0000000 -size 0x1000  ( This is the size of our space 4K)

- Do one write sequence

- $: mwr 0xA0000000 0x00000003  ( Reg 0 : This sets write enable)

- $: mwr 0xA0000004 0x00080000  ( Reg 1: Upper 16 bit are for Mem A)

- $: mwr 0xA0000008 0x12345678  ( Reg 2: Our Data)

- Do one read back sequence

- $: mwr 0xA0000000 0x00000004  ( Reg 0: This sets read enable)

- $: mwr 0xA0000004 0x00080000  ( Reg 1: Upper 16 bits are for Meme A)

- $: mrd  0xA000000C  ( Reg 3: This should return 0x12345678)